******************************************************************************

*				IMPORTANT				     *

*									     *

*	If you have:							     *

*		a) a Compaq Video Graphics Controller Board with	     *

*		   ROM revision A.2 or less, and,			     *

*		b) a MicroSoft Mouse with Version 6 of the driver	     *

*	then you have a potentially serious problem.			     *

*									     *

*	A bug in this configuration of the system may cause PSpice or	     *

*	ANY other program to destroy the data on your hard disk.	     *

*									     *

*	You can either:							     *

*		a) obtain patch 'INT10VGA.SYS' from your Compaq dealer, or   *

*		b) unplug your mouse, then reboot the system prior to	     *

*		using our software.					     *

*									     *

*	Should you fail to take corrective action and destroy your disk,     *

*	rest assured that you have our sympathy.			     *

******************************************************************************

INSTALLATION

1.1  PSpice Installation for the IBM-PC Evaluation Package using DOS

The following sections give instructions for installing PSpice on

the IBM-PC family of computers.

1.2  System Requirements

PSpice will run on any IBM-PC/XT/AT, or PS/2 with 640-kilobytes of memory,

a fixed disk, and the MS-DOS 3.0 (or later) operating system.  An

8087/'287/'387 floating-point coprocessor is optional (for the evaluation

version only), but with Probe and Parts, traces will be drawn 5 to 10 times

faster with the coprocessor.  Either the monochrome or the color graphics

display may be used.  No special brand of printer or special printer features

are needed.

PSpice will also run on IBM-compatible systems, as long as they meet

the above requirements.  The program has been tested on different

hardware configurations and it was found that if the system will run

MS-DOS 3.0, has the memory, and has the floating-point coprocessor,

then it will run PSpice.  Note that PSpice is delivered on IBM-format

diskettes.

1.3  Doing the Installation

In the package which you received, you should find:

1)  The PSpice user's guide.

2)  A diskette containing these files:

CONFIG.SYS

CSHELL.DOC

EXAMPLE1.CIR

EXAMPLE1.OUT

INSTALL.DOC

NOM..IND

NOM..LIB

PARTS.EXE

PARTS.HLP

PREVL421.OVL

PREVL422.OVL

PREVL423.OVL

PROBE.DEV

PROBE.EXE

PS..EXE

PSPICE.BAT

PSPICE.NDX

PSPICE1.EXE

README.DOC

STMED.EXE

STMED.HLP

3)  A diskette containing these files:

PSEVL421.OVL

PSEVL422.OVL

PSEVL423.OVL

SEEVL421.OVL

PSPICE.HLP

Installing PSpice, Probe, Parts, and the Stimulus Editor consists mainly of

making working copies of these files.

Before beginning, print and read the file README.DOC.  It

contains last-minute information which did not make it into

the user's guide.

1.4  Installation on a Fixed Disk.

The installation is simple: copy all the files from all the diskettes onto

the fixed disk in the directory where you normally keep your program files.

If you do not have a CONFIG.SYS file in your root directory,

copy CONFIG.SYS from the PSpice diskette into your root directory.

If you do already have a CONFIG.SYS file, examine the one on your PSpice

diskette and add any statements which it contains to yours e.g.,

BUFFERS=20, and FILES=20;

Print EXAMPLE1.OUT.  Later, you will want to compare it against

the result of running EXAMPLE1.CIR.

The diskettes are your backup copies and should be stored in a safe

place.

1.5  Comments

PSpice is composed of one program file, PSPICE1.EXE, and several

overlay, .OVL, files.  You can put the PSPICE1.EXE and overlay files

in any directory you choose, as long as you follow these guidelines:

1)  Do not rename any of the .EXE files or the .OVL files.

The overlay handler will not find overlays unless their names are

as we ship them.

2)  Make sure that the overlay files are in a drive and directory

listed in your system's PATH command.  Normally the PATH command

is included in the file AUTOEXEC.BAT in your system's root

directory or on your system (boot) floppy.

The CONFIG.SYS file in the ROOT directory configures the system when it

is booted.  The BUFFERS=20 statement gives PSpice enough disk buffering to

work efficiently, and the FILES=20 statement allows PSpice to

open all the files that it needs.